CS-350 Final Project: SysTec Smart Thermostat Architecture Report

Steven V. Burgett

Department of Science, Southern New Hampshire University

CS-350: Emerging Sys Arch & Tech

Professor Christopher Kardell

August 16, 2026

The SysTec prototype reads the AHT20 sensor through I2C, controls LEDs and buttons through GPIO/PWM, updates a 16x2 LCD, and sends status through UART. A production design must retain these peripherals while adding Wi-Fi and enough memory for cloud communication.

Architecture Comparison

Raspberry Pi 4 offers the largest development environment. It includes a 40-pin GPIO header, onboard dual-band 802.11ac Wi-Fi, and 1 GB to 8 GB of RAM (Raspberry Pi Ltd., n.d.). Its interfaces readily support the thermostat's GPIO, I2C, and UART needs, and Linux provides strong networking and software support. The disadvantage is that this level of processing power and memory is more than a dedicated thermostat requires, adding unnecessary complexity for a production appliance.

Microchip provides a more focused embedded option. The PIC32MZ-W1 Wi-Fi MCU family integrates wireless networking with up to 2 MB of Flash, 640 KB of RAM, and up to 60 GPIOs (Microchip Technology Inc., n.d.). That capacity is appropriate for buttons, LEDs, display control, the I2C temperature sensor, UART communication, and cloud connectivity. Because the application runs directly on a microcontroller, this approach can reduce system overhead compared with a Raspberry Pi while still meeting the project's functional requirements.

For the Freescale architecture, the current NXP portfolio represents the successor platform. The NXP RW612 is a low-power wireless MCU that integrates an Arm Cortex-M33 processor with Wi-Fi 6, Bluetooth Low Energy, and 802.15.4, and NXP identifies connected smart-home and smart-energy devices as target applications (NXP Semiconductors, n.d.). Its embedded peripheral support and integrated wireless capability make it well suited to the same GPIO, sensor, display, and serial functions demonstrated by the prototype without requiring a full Linux computer.

Recommendation

I recommend the NXP/Freescale wireless MCU architecture, specifically the RW612 class. Raspberry Pi 4 remains an excellent prototype platform but is oversized for a dedicated thermostat. Microchip is also a strong production option because its Wi-Fi MCUs combine adequate memory, GPIO, and wireless connectivity. The RW612 best balances integrated Wi-Fi, embedded performance, peripheral support, low-power operation, and direct suitability for connected smart-home products.

References

Microchip Technology Inc. (n.d.). Wi-Fi microcontrollers (MCUs). https://www.microchip.com/en-us/products/wireless-connectivity/embedded-wi-fi/microcontrollers

NXP Semiconductors. (n.d.). RW612 wireless MCU with integrated Wi-Fi 6, Bluetooth Low Energy 5.4, and 802.15.4. https://www.nxp.com/products/wireless-connectivity/wi-fi-plus-bluetooth-plus-802-15-4/wireless-mcu-with-integrated-tri-radio-1x1-wi-fi-6-plus-bluetooth-low-energy-5-4-802-15-4:RW612

Raspberry Pi Ltd. (n.d.). Raspberry Pi 4 Model B specifications. https://www.raspberrypi.com/products/raspberry-pi-4-model-b/specifications/